#include <Wire.h>

#include <LiquidCrystal_I2C.h>

#include <Adafruit_MPU6050.h>

#include <Adafruit_Sensor.h>

#include <TinyGPS++.h>

#include <SoftwareSerial.h>

#include <math.h>

// ---------- Pins ----------

#define LCD_SDA_PIN   8

#define LCD_SCL_PIN   9

#define IMU_SDA_PIN   2

#define IMU_SCL_PIN   3

#define BUZZER_PIN    20

#define BUTTON_PIN    15

#define MQ2_PIN       16

#define TRIGGER_PIN   14

LiquidCrystal_I2C lcd(0x27, 16, 2);

Adafruit_MPU6050 mpu;

// ---------- GPS ----------

SoftwareSerial gpsSS(4, 6);

TinyGPSPlus gps;

// ---------- GSM ----------

#define EMERGENCY1 "+917022880385"

#define EMERGENCY2 "+919108735137"

#define EMERGENCY3 "+919740166569"

// ---------- SETTINGS ----------

#define CRASH_THRESHOLD_G 1.0

#define ALERT_COOLDOWN 15000

unsigned long lastAlertTime = 0;

bool lastDrowsyState = LOW;

// ---------- LCD ----------

void lcdPrintLine(uint8_t row, const char* s) {

lcd.setCursor(0, row);

lcd.print("                ");

lcd.setCursor(0, row);

lcd.print(s);

}

// ---------- BUZZER ----------

void beep() {

digitalWrite(BUZZER_PIN, HIGH);

delay(700);

digitalWrite(BUZZER_PIN, LOW);

}

// ---------- GPS ----------

void updateGPS() {

while (gpsSS.available()) gps.encode(gpsSS.read());

}

// ---------- MESSAGE ----------

String buildMessage(const char* type, float peak) {

String msg = "EMERGENCY DETECTED !\r\n";

msg += "Type: ";

msg += type;

msg += "\r\n";

if (peak > 0) {

msg += "Impact: ";

msg += String(peak, 2);

msg += "g\r\n";

}

// ✅ LIVE GPS

if (gps.location.isValid() && gps.location.age() < 2000 &&

gps.location.lat() != 0.0 && gps.location.lng() != 0.0) {

double lat = gps.location.lat();

double lon = gps.location.lng();

msg += "Lat: ";

msg += String(lat, 6);

msg += "  Lon: ";

msg += String(lon, 6);

msg += " (live*)\r\n";

msg += "https://maps.google.com/?q=";

msg += String(lat, 6);

msg += ",";

msg += String(lon, 6);

msg += " (live*)";

}

else {

msg += "GPS : NO FIX";

}

return msg;

}

// ---------- CALL ----------

void makeCall(const char* number) {

lcdPrintLine(0, "CALLING...");

lcdPrintLine(1, number);

Serial1.print("ATD");

Serial1.print(number);

Serial1.println(";");

delay(8000);

Serial1.println("ATH");

delay(1000);

}

// ---------- SMS ----------

void sendSMS(const char* number, String msg) {

Serial1.print("AT+CMGS=\"");

Serial1.print(number);

Serial1.println("\"");

delay(300);

Serial1.print(msg);

Serial1.write(26);

delay(2000);

}

// ---------- EMERGENCY ----------

void triggerEmergency(const char* type, float peak) {

if (millis() - lastAlertTime < ALERT_COOLDOWN) return;

lastAlertTime = millis();

lcd.clear();

lcdPrintLine(0, "*** ALERT ***");

lcdPrintLine(1, type);

beep();

String msg = buildMessage(type, peak);

Serial1.println("AT");

delay(100);

Serial1.println("AT+CMGF=1");

delay(100);

makeCall(EMERGENCY1);

sendSMS(EMERGENCY1, msg);

sendSMS(EMERGENCY2, msg);

sendSMS(EMERGENCY3, msg);

lcdPrintLine(0, "SOS SENT!");

lcdPrintLine(1, type);

delay(1500);

}

// ---------- SETUP ----------

void setup() {

Wire.setSDA(LCD_SDA_PIN);

Wire.setSCL(LCD_SCL_PIN);

Wire.begin();

lcd.init();

lcd.backlight();

lcdPrintLine(0, "System Ready");

lcdPrintLine(1, "Initializing...");

delay(2000);

lcd.clear();

Wire1.setSDA(IMU_SDA_PIN);

Wire1.setSCL(IMU_SCL_PIN);

Wire1.begin();

if (!mpu.begin(MPU6050_I2CADDR_DEFAULT, &Wire1)) {

lcdPrintLine(0, "MPU ERROR!");

while (1);

}

pinMode(BUZZER_PIN, OUTPUT);

pinMode(BUTTON_PIN, INPUT_PULLDOWN);

pinMode(MQ2_PIN, INPUT);

pinMode(TRIGGER_PIN, INPUT);

Serial1.setTX(0);

Serial1.setRX(1);

Serial1.begin(115200);

gpsSS.begin(9600);

gpsSS.listen();

}

// ---------- LOOP ----------

void loop() {

updateGPS();

// ---- CRASH ----

sensors_event_t a, g, t;

mpu.getEvent(&a, &g, &t);

float mag = sqrt(a.acceleration.x * a.acceleration.x +

a.acceleration.y * a.acceleration.y +

a.acceleration.z * a.acceleration.z);

float crash_g = fabs((mag / 9.80665) - 1.0);

char l0[17], l1[17];

snprintf(l0, sizeof(l0), "G:%5.2f g", crash_g);

snprintf(l1, sizeof(l1), "Thresh:%4.1f", CRASH_THRESHOLD_G);

lcdPrintLine(0, l0);

lcdPrintLine(1, l1);

if (crash_g >= CRASH_THRESHOLD_G) {

triggerEmergency("CRASH", crash_g);

}

// ---- SOS ----

if (digitalRead(BUTTON_PIN) == HIGH) {

triggerEmergency("SOS", 0);

}

// ---- GAS ----

if (digitalRead(MQ2_PIN) == LOW) {

triggerEmergency("GAS DETECTED", 0);

}

// ---- DROWSINESS ----

bool currentDrowsyState = digitalRead(TRIGGER_PIN);

if (currentDrowsyState == HIGH && lastDrowsyState == LOW) {

delay(50);

if (digitalRead(TRIGGER_PIN) == HIGH) {

triggerEmergency("DROWSINESS", 0);

}

}

lastDrowsyState = currentDrowsyState;

delay(100);

}